VT52  is  a VT52 terminal emulator written for Kaypro II and  IV  (non-

graphic versions).  It contains the following features:

1) Interrupt driven

2) 256 byte receive buffer

3) automatic flow control

4) direct to video emulation of VT52

5) Supports application keypad

6) Baud rate programming

7) Break and null generation

8) Additional escape sequences to support EMACS

Because  of 1 through 3,  VT52 is able to run at high baud rates  (I've

used 1 it at 4800) without losing characters.   This is true even on non-sped

up Kaypros.

Because  of  4,  VT52 does a much better job of emulating a  VT52  than

other  programs that only translate VT52 escape sequences into Kaypro escape

sequences (for example reverse line feed on VT52).

Keypad  application  mode  is selectable via  the  normal  VT52  escape

sequences received from a host computer.   The keypad on the Kaypro does not

have all the keys necessary to map into the VT52 keypad,  so I had to choose

which keys to map where (see KAYPRO.KEY).   The only key that is lost is PF2

(normally  the help key) and this can be mapped to a control key when in  an

editor such as VAX's EDT.

All features such as baud rate selection,  break generation are invoked

in terminal mode of VT52 by typing ^\ followed by the appropriate key.   For

a list of the commands, type ^\ ? in terminal mode.

The  escape  sequences  that VT52 has in addition to  the  normal  VT52

escape sequences were chosen to allow a much faster screen edit using EMACS.

The escape sequences are:

ESC @ 1        - Insert space at cursor (cursor stays)

ESC @ 2        - Delete character at cursor

ESC @ 3        - Insert line at cursor line

ESC @ 4        - Delete line at cursor line

There  are  some area's that still could be improved  in  VT52.   Since

source is included,  I invite anyone to do so.  At present, VT52 does not do

auto  wrap.   It  acts like a VT52 with auto wrap off (cursor stays in  last

column).  This presents no problem when communicating with VAX/VMS.  The VAX

can be set to do the wrapping for the terminal.   Also,  VT52 printer escape

sequences are not emulated.   That might be a nice feature (but I can't  use

it so I didn't bother).

VT52 is written in Z80 code.   This was necessary to support the mode 2

interrupts  used for high speed.   Block move instructions are also used  in

the  video  code.   To assemble,  I used Z80MR,  a public domain  Z80  macro

assembler  (available  for Micro-Cornucopia,  P.O.  Box  223,  Bend,  Oregon

97709).   To re-assemble,  type Z80MR VT52.AAA (similar to ASM).   The other

source files are INCLUDED by VT52.AZM.

If anyone has problems, suggestions, questions, etc. send them to:

Ray Rizzuto

43 Sandra Road

Voorhees, N.J. 08043

Compuserve ID 72765,700

######################################################################################